Name:…………………………………………………………………………………     Index. No. ………………………..

Adm. No……….Class: …………

Signature:…………………………

232/3

PHYSICS								Date: ………………………………..

PRACTICAL

JULY, 2018

TIME: 2 ½ HRS

M O K A S A    II   J O I N T   E X A M I N A T I O N - 2018

Kenya Certificate to Secondary Education

PHYSICS PAPER 3

PRACTICAL

Instructions

- Write your name, admission number, class, signature and date of examination in the spaces provided at the top of the page.

- Answer all the questions in the spaces provided in this paper.

- You are supposed to spend the first 15 minutes of the 2 ½ hours allowed for this paper reading the whole paper carefully before your start.

- Marks will be given for clear record of observations actually made, for their suitability and accuracy, and the use made of them.

- Candidates are advised to record their observations as soon as they are made.

- Electronic calculators and mathematical tables may be used.

FOR EXAMINER’S USE ONLY

Question 1

|  | (a) (i)-(v) | (b) | c(i) | (c) (ii)I-II | (d)(i)-(iv) | (ii) | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Maximum Score | 2 | 6 | 5 | 3 | 2 | 2 | 20 |
| Candidate’s Score |  |  |  |  |  |  |  |

Question 2

| PART A | (a)(i)-(iii) | (b) | (c)(i)-(iii) | D (i)-(iii) | PART B | (a) | (b) | (c) | (d) | TOTAL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Maximum Score | 3 | 1 | 2 | 7 |  | 1 | 4 | 1 | 1 | 20 |
| Candidate’s Score |  |  |  |  |  |  |  |  |  |  |

This paper consists of 9 printed pages.  Candidates are advised to check and to make sure all pages are printed.

Q1.	(a)	You are provided with the following apparatus.

Question 1.

- A metre rule

- A mass marked M

- Six 20g masses

- A stop watch

- A complete stand

- A balance for sharing

Arrange the apparatus as shown in the figure 1.1 (a) below.

(a) (i) Record the Pointer reading R0 of unloaded spring, R0 = ……………………cm.

(½ mark)

Attach the mass marked M to the free end of the spring to exert a downward force Mg in newtons as shown in figure 1.1 (b) above.  If the mass causes an extension e called static extension, then Mg = ke where k is the spring constant and g is acceleration due to gravity. (g = 10N/kg)

(ii) Record the pointer reading R for the mass M, R = …………………..…cm.

(iii) State the static extension e caused by M in cm.

e  = …………………………………………………………………………….. cm.	    (½ mark)

(iv) Using the balance, weigh the mass of M in grams ………………g.

(½ mark)

(v) Determine the spring constant k in newtons per meter.     	   (½ mark)

(b) Pull the mass M down a further distance x below the equilibrium position as shown in figure time 10 small oscillations and record your values in the table shown below.

Repeat the experiment for other pointer reading R and complete the table.

| Mass M(g) | Pointer Reading, R(cm) | Extension e (cm) | Time for 10 oscillations t (s) | Period T (s) | T2 (s2) |
| --- | --- | --- | --- | --- | --- |
| O |  |  |  |  |  |
| M |  |  |  |  |  |
| M + 50 |  |  |  |  |  |
| M + 100 |  |  |  |  |  |
| M + 150 |  |  |  |  |  |
| M + 200 |  |  |  |  |  |
| M + 250 |  |  |  |  |  |

(6 marks)

(c) (i) Plot the graph of e (y – axis in metres) against T2 on the graph paper provided.  								       	(5 marks)

Hint: Draw your axes to include point (0, 0) a third way up the page i.e., T2 axis to run across a third of the page from the bottom.

(ii) From the graph determine;

(I) The slope of the graph, S.					(2 marks)

(II) The intercept of the e-axis, C.				(1 mark)

(d) If Ms is effective mass of the spring, the period T of the oscillating system is given by;

e=g.T24π2-gMsk

Using this formula, calculate:

- Acceleration due to gravity g.				(2 marks)

- The effective mass Ms of the spring.			(2 marks)

Question 2.

PART A

You are provided with the following;

- A micrometer screw gauge ( to be shared)

- A centre zero galvanometer

- A switch

- Ten connecting wires (at least five with a crocodile clip on one end)

- A resistance wire mounted on a millimeter scale labelled AB

- A resistance wire labelled P

- A resistance wire labelled Q

- A metre rule or half metre rule

- Two dry cells and a cell holder.

Proceed as follows:

- Using the micrometer screw gauge provided, measure the diameter.

- D of wire P.

D = …………………………..					(1 mark)

- Determine cross-sectional area Ap of wire P (in metres2)

AP= …………………..						(1 mark)

- d of wire Q

d = …………………………….					(1 mark)

- Determine C1 the value of the ratio  Dd.

C1  = …………………………………………………….				(1 mark)

- Set up the circuit as shown in the figure below.  (Ensure that each of the

wires P and Q is 50cm long)

- Close the switch.  Using the clip at the free end of the wire from the galvanometer, tap wire AB near end A and observe the deflection in the galvanometer.

- Then tap the wire near end B and again observe the deflection in the galvanometer. Check your circuit if it does not deflect in the opposite direction.

- Now tap the wire AB at various points between A and B to obtain a point K where there is no deflection in the galvanometer.

- Determine the length L1, the distance from A to K.      (1 mark)

L1 = ………………………………………..

- Determine the length L2, the distance from B to K.     (1 mark)

L2  = ………………………………………

- (i)	Given that the resistance RQ of Q is 9.0 ohms, determining the

resistance Rp of P using the expression: 			    (2 marks)

RPRQ=L1L2

(ii) Determine the value of C2 given that;					(2 marks)

C2= RQRP

(iii) Compare the value of C1 (in part (b) with that of C2.			(1 mark)

- Determine the resistivity pof wire P given that the resistance Rp= p.LpAp

(2 marks)

PART B

You are provided with the following:-

- Convex lens on a lens holder, L

- A candle, O

- Screen, S

- Metre rule

(a)	Use a distant object to determine the focal length, f1.

f1  = …………………………………cm						(1 mark)

(b)

- Set the apparatus as shown.

- Set the distance X to, X = 70cm and lens position at L1 and closer to the object O.  Move the lens towards the screen S until an enlarged image is focused on S.

Record the distance U1.

- Move the lens to position L2 and closer to the screen, where a sharp diminished image is focused on the screen.

Record the new distance U2.

- Repeat for the other given value of distance X and complete the table.

Note:  The focal length f is given by the equation f= x2- d24x

| X (cm) | U1 (cm) | U2 (cm) | d=U2-U1cm | f=x2-d24x cm |
| --- | --- | --- | --- | --- |
| 70 |  |  |  |  |
| 50 |  |  |  |  |

(4 marks)

(c)	From the above measurements determine the average focal length f2 of the lens.

(1 mark)

f2   =

(d)	Calculate the ratio R of the focal length obtained in (a) to the focal length obtained in (c) above.  									(1 mark)

R= f1f2  =