PHYSICS

232/2

MARKING SCHEME

TERM 1 2019

SECTION A (25 MARKS)

Rays and all angles shown 3 marks

sin42.5=1n

n=142.5=1.48

- Soft iron became induced magnet and attracts back the razor

Wood is non-magnetic material hence does not attract the blad

- The charges that accumulates on the body of the car by rubbing with the air leak to the earth through the man while he stept out

- A Tv tube use magnetic coils for deflection of an electron beam instead of electric field .Magnetic field gives a wider deflection

- i)	Diffraction1

ii) 	Interference.1

The two opening acts as two source1 hence 	the two waves interfere producing soft and 	loud sound along the way1

- The leaf diverges

Negative charges from electroscope flows to the sphere leaving +ve charges on the electroscope thus there is divergence.

- E = IR + Ir

3.0 = I ×3.5 ×I ×0.51

∴ I = 0.75A;

- A thick sheet of plastic, n = 1.5, is used as the side of an aquarium tank. Light reflected from a fish in the water has an angle of incidence of 350. At what angle does the light enter the air.	(3 mks)

n; sin ө; = nr sin өr

1.3 sin 350 = 1.5 sin өr

өr   =   29.80

sin r = sin i

n

=  sin 29.8

1.5

=  0.3313

r   =  19.39

- When the object distance from the pinhole is 	equal to the image distance.

- When the screen is as large as the object1(any 1) Give 1 mk

11.	Period = 0.033  = 0.011

f = 1T = 10.011

f = 100Hz

12.The figurebelow shows a laclanche cell.

A  Manganese Iv Chloride

B Ammonium IV chloride

Section b

13. (a)													(4 Marks)

- Correct diagrams.

- It occurs when the earth is between the sun and moon and all lie on a straight line.

- The earth casts a shadow on the moon.

(b)	For security reasons since they cover a wide field of view.

(c)	hiho = vu												(3 Marks)

hi2.5 = 0.168

hi = 0.16 x 2.58 = 0.05 or 5cm

(d)	(i)

(ii) n1sin1 = n1sin2										(3 Marks)

32SinC = 43Sin90

C = Sin-10.8889

= 62.720

14. (a) 	Capacitance is charge per unit volt

(b) 	(i) 2μ₣ + 6μ₣ = 8 μ₣

Effective capacitance =Product

sum

= 8 x 3

8 + 3

= 24

11

= 2.182 μ₣

    - Total charge Q = CV = 2.18 x 10-6 x 12

= 26.16 x 10-6C

(iii) p.d across the 2 μ₣ = 26.16 x 10-6√1

8 x 10-6

= 3.27V

- Ohm’s law – The amount of current flowing through a metallic conductor is directly proportional to the p.d across its ends so long as temperature and other physical conditions remain constant.

(ii)	Effective resistance

RE 	= 4 + 6 x 3

3 + 6

= 4 + 18

9

= 6Ω

V =IR

I  =12 = 2A

6

. V = 2 x 4 = 8V

(ii) 	V = 12 – 8 = 4V√

. V = 1R

I = 4 = 2 = 0.667 A√

6    3

15. (a)	(i) 	Frequency not affected. 1

(ii)	Speed reduces. 1

(iii)	Wavelength reduces. 1

(b)	f = v = 0.12 = 1.5 Hz 1

 0.08

(c)	Gamma ray, x-rays, ultraviolent, visible light, infra red, microwaves.

(d)	Stationary wave	Progressive ways

1. No energy is transferred   Energy is transferred from

from source.		        source. 1

2.Wave form does not 	        Wave form moves away

appear to move.	         continuously. 1

(e)	Wave length = 9/6 = 1.5 cm 1

16. (a)	(i) 	At A light undergoes refraction / dispersion1/2

At B light undergoes total internal reflection1/2

(ii) 	C - Violet

D - Red

17. . a)	-	Axes labeled with units; 1

-	Consistent scale ;	1

-	5 points2 ( 3 – 4 points ) 1

-	Line ; 1

Voltage(V)

Resistance = gradient

= 2.60 – 1.00

(350 – 150) x 10-3

= 8.0   0.2

Current(mA)

- Resistance = slope of Graph ;

= 2.2 – 1.4

0.3 – 0.2

= 80hrs;

Hhhhhh